NZIPL Industry Roundtable: Solar PV

Key Takeaways

Background:

On February 25, 2026, the Net Zero Industrial Policy Lab organized two industry roundtable sessions, with a sectoral focus on solar PV and BESS in New Delhi, India. The…

Participation:

Overview:

India’s renewable energy landscape has transformed over the past decade. This transformation has been led in part by scaling solar capacity. India’s installed solar capacity rose from about 3 GW in 2014 to 140 GW in January 2026. Table X highlights key trends in India’s clean energy landscape.

INSERT TABLE X

A driving factor

Key Takeaways:

Key inputs, machinery & materials:

- Machinery: Domestic capabilities and deficiencies in machineries and metals crucial to the solar value chain need to be mapped.

- Perovskite: There are incremental changes with perovskites. Tandem solar cells have resulted in efficiency rates above 30%. This has not been commercialized yet and is limited to labs or at the R&D stage at various firms.

- Key components: Participants highlighted the importance of components such as silver paste, junction boxes, and diamond wire saws.

  - Silver paste: This is a critical component in solar cell manufacturing (due to its superior electrical conductivity) but materials research for this product is limited. A senior research scientist highlighted an example (from 2016) wherein extensive R&D was conducted for silver at a leading institution but received no support from the government or industry despite repeated requests.

  - Junction boxes: These are critical components and there are greater demands for higher-rated current junction boxes. While there is growing indigenous capabilities for this component, industry should strengthen its domestic manufacturing capabilities.

Manufacturing:

A leading manufacturer of solar modules stated that key decisions regarding the manufacturing of their products hinges on three factors: efficiency, energy, and cost.

- Manufacturing ecosystem: There are limitations when it comes to equipment vendors for key machineries in the process and value chain. The government, industry, academia and others should actively map the ecosystem identifying deficiencies and dependencies.

- Design & fabrication capabilities: Module assembly capabilities in India are currently well-established and cell fabrication capabilities are increasing. Stakeholders need to invest in bolstering domestic design and fabrication capabilities.

- Falling costs: The broader implications for the domestic industry need to be accounted for in the scenario where the LCOE for solar PV continues to fall.

Policy & regulatory landscape:

- National roadmap: The domestic industry has strengthened considerably over the past decade. The government should consider creating a national roadmap for the solar industry over the medium to long term to provide policy signals to the industry and direct R&D efforts.

- Approved List of Models and Manufacturers (ALMM) scheme: This scheme has been crucial in building up the domestic industry and further modifications to the scheme (including the proposed addition of solar wafers starting in June 2028) are important. The ALMM scheme should be regularly fine-tuned following consultations between the industry and government.

  - The government should consider instituting an ALMM scheme for key ancillary products in the solar module value chain.

- Production-Linked Incentives (PLI) scheme: Select participants pointed out that the PLI scheme favors large, incumbent players and instead of a production-defined scheme, the government could consider instituting a ‘Roadmap-Linked Incentives’ (RLI) scheme. A RLI scheme would equalize the playing field allowing for firms that achieve certain set milestones along a roadmap (defined by a combination of domestic content add, R&D investment, capital investment, manufacturing capacity etc.)

  - Other participants pointed out that solar manufacturing is a game of scale and given the China challenge, a PLI-like scheme that enables large players is better-suited for the Indian market.

- Distribution-focused policies: Over the last few years, the government has instituted policies that focus on the distribution side which has in turn shaped the domestic solar manufacturing industry. Notable policies include PM-SURYA and PM-KUSUM.

  - Participants highlighted state-level policies, including strides made by the Maharashtra State Electricity Distribution Company Limited (MSEDCL, also known as Mahavitaran)…

R&D:

- Reliance on foreign researchers: A senior research scientist pointed out that domestic firms continue to seek external experts for key R&D and manufacturing projects.

- Funding: Scientists reflected on the transformed funding landscape in India. Participants noted that the quantum of and access to funding R&D projects used to be a challenge previously, this is not the case anymore.

- Firms: Private firms invest none to limited resources towards research and design.

  - A major producer of solar modules stated that their firm is actively investing in startups across the solar value chain in India.

- Government research institutions: Key research institutes highlighted during the discussion include the National Centre for Photovoltaic Research and Education (NCPRE) at the Indian Institute of Technology, Bombay (IIT-B) and the Indian Institute of Science (IISc).

  - IISc established the ‘Solar PV Outdoor Reliability Testbed’ (one of the first such large-scale research facilities dedicated to “understanding the real-world performance and reliability of solar PV technologies”) at its Challakere campus in 2010…

  - There are several start-ups at IIT-B that focus on different aspects of the value chain including solar cells, battery storage systems, and silver.

- Process innovation & material R&D: Increasing investments in process innovation, design R&D, and materials research is crucial. Process innovation occurs as much on factory floors as it does at R&D labs. This necessitates collaboration between academia, government, and industry.

  - Industries that are aligned with firms and products in the solar value chain could potentially partner and work with the government to invest in process innovation and materials research. Examples include…

- Next gen technologies: Participants emphasized the importance of investing in next gen technologies that could enable Indian firms to leapfrog key competitors. This entails identifying frontier technologies and key R&D projects across the solar value chain, and charting avenues for academia, government, and industry to work together in this domain.

- Purchasing patents: Participants questioned the potential for industry actors to purchase patents instead of investing in R&D. This could be cost-saving in the short-term for resource-limited firms.

- Pilot manufacturing: Participants questioned the cost competitiveness of conducting pilot projects in India, relative to other countries. Some argued that industry players should focus on the design aspect of R&D given the relative gains.

- CSR models for R&D: Participants recommended novel R&D funding mechanisms, including a CSR-style model for R&D purposes.

Circularity:

- Emissions intensity: Participants highlighted the energy-intensive nature of manufacturing PV panels and underscored the importance of building up circularity in the value chain. The production processes for certain forms of solar modules have greater emissions intensity.

China factor:

- Trade practices: In today’s geopolitical climate, trade policy is climate policy.

- Comparisons between India and China: The solar industry in China has been nurtured over XX years. Comparisons between the industries in India and China need to be done keeping in mind the differing timelines of the policies instituted, and the resources invested in the two countries.

  - For instance, provide the example of how ALMM has been around for XX /18 months and China has devoted 10 years to XX policies.

- Chinese solar PV industry: The Chinese industry, while dominant, is full of loss-making companies that are supported by local and state governments. A central challenge for Indian policymakers is to figure out how to create and protect domestic industries.

- Boom and bust cycles: The Chinese industry undergoes boom and bust cycles, and it is crucial for stakeholders across the value chain to keep track of movements in the industry to identify opportunities and headwinds in global demand and supply.

Skill building:

- Next generation of technologists: Industry and government need to work together to identify how to train the next generation of technologists for the solar industry. Given the industry’s growing importance in India’s energy landscape, identifying skill-training curriculum, programs, and institutes is crucial.

Key trends:

- RE uptake by C&I: C&I, and the metals industry in particular, are transitioning to greater renewable energy due to increasing cost competitiveness. This will impact RE demand and the solar manufacturing industry.

- Transmission challenges: A key challenge over the short to medium term pertains to grid challenges. Participants noted that on average RE projects take less time to build out (for instance, thermal coal projects take 5 years whereas solar takes 18 months) and this has resulted in ….

Silver paste is a key material for photovoltaic panels, with the PV sector accounting for 17% of total silver demand in 2025 (at 196 million troy ounces). In 2025, silver rallied 130% (reaching highs of $121.64 an ounce) with senior consultants estimating that the metal accounts for one of the greatest contributors to the increased cost of manufacturing panels. This price rise is in part due to U.S. tariff policy in 2025. Firms are responding to this price rise in two ways. The first is to seek alternatives such as copper (which trades at $12,823 per metric ton compared to silver at $2.5 million per ton). An executive focusing on the solar supply chain expects “broader industry shifts” in 2026 with “leading manufacturers moving to pure copper metallization and hybrid silver-copper pastes.” Manufacturers could also turn to silver-free technology such as cadmium telluride thin-film technology. Market analysts expect these trends to play out over years. Firms are also offsetting raw material volatility through technology gains and value addition. A leading Indian manufacturer noted that silver paste accounts for a smaller share of total cell costs in 2026 compared to previous years due to “sustained reductions in consumption through process innovation and R&D.” Also see: https://onlinelibrary.wiley.com/doi/full/10.1002/pip.3661; https://www.sciencedirect.com/science/article/pii/S0921344923002823; https://www.ipmi.org/symbol?tvwidgetsymbol=TVC%3ASILVER; https://payneinstitute.mines.edu/solar-surge-puts-pressure-on-silver-supply/; https://www.sciencedirect.com/science/article/abs/pii/S0956053X16301015?via%3Dihub; https://pv-magazine-usa.com/2026/02/27/the-richest-silver-mine-isnt-underground-its-in-solar-panels/

In the latest MNRE drafts, the ministry mandates that government-backed ALMM projects should source modules from List-I manufacturers, cells from List-II and wafers from forthcoming List-III firms. The policy targets increased domestic production across the value chain. The proposed List-III for solar wafers will be published after a minimum of three independent manufacturers have set up wafer facilities and collectively reach annual capacity of at least 15 GW. See: https://www.pv-tech.org/indias-solar-manufacturers-applaud-almm-proposal-boost-domestic-wafer-production/

A participant recounted their visit to the ‘State Key Laboratory of PV Science and Technology’ located in Changzhou, China. The lab is housed within the headquarters of Trina Solar, a leading Chinese manufacturer. The lab focuses on “investigating novel mechanisms for achieving high-efficiency photovoltaic conversion in crystalline silicon cells and advancing industrialization” and comprises more than 200 permanent members, setting “32 world records in both crystalline silicon cell efficiency and module power.” The participant noted this experience to highlight an example of academia, industry, and government collaboration. Also see: https://www.trinasolar.com/en-glb/innovation/; https://www.prnewswire.com/news-releases/trina-solars-state-key-laboratory-of-pv-science-and-technology-receives-ministry-accreditation-231419301.html; https://en.sklpvst.com/about; https://www-cdn.trinasolar.com/wwwstorage/sites/3/TrinaSolar_CSRreport_ESG_2021_EN.pdf; https://www.motherjones.com/politics/2018/06/solar-china-donald-trump-xi-jinping-innovation-huawei-1/.

https://www.ceew.in/sites/default/files/how-can-india-enable-circular-economy-with-solar-waste-management.pdf

According to the OECD MAGIC database on industrial subsidies, “the production of solar cells and modules was, on average, the most subsidized between 2005 and 2024, with subsidies representing nearly 3.2% of firms’ revenue against and overall MAGIC database average of 0.9%.” This has resulted in “sustained imbalances between supply and demand and financial hardship for the industry.” Previous research tracking China’s industrial policy spending highlighted that this spending came out to at least 1.73% of its GDP in 2019 (using a conservative methodology) at $248 billion at nominal exchange rates. To put this in perspective, this figure is higher than its defense spending for 2019. In addition to expansive government support, leading solar manufacturers in China have engaged in deflationary price wars, with firms selling at below cost to undercut competitors. Leading Chinese solar firms’ net income has been in the red since 2023 and the Chinese government is concerned about “involution” (excess production leading to a race to the bottom in prices). In 2024, China’s biggest solar firms (Longi Green Energy, Trina Solar, Jinko Solar, JA Solar and Tongwei) shed around 87,000 staff, or almost one-third of their total workforces, and since then over 40 solar firms “have delisted, gone bankrupt, or been acquired” according to the photovoltaic industry association. In July 2025, executives from the biggest Chinese solar firms were summoned to Beijing where Li Lecheng, the industry minister, said companies “must “resolutely crack down” on disorderly competition and speed up the closure of underutilized factories.” Beijing also instituted policies, like placing limits on the energy consumption of polysilicon plants in 2025, that was forcing less efficient capacity to shut down. In 2025, China’s five leading PV companies posted deficits, with total losses ranging from RMB28.9-32.8 billion (US$4.1-4.7 billion).

Also see: https://www.intereconomics.eu/contents/year/2024/number/4/article/eu-concerns-about-chinese-subsidies-what-the-evidence-suggests.html; https://rhg.com/research/china-and-the-future-of-global-supply-chains/; https://dialogue.earth/en/business/chinese-solar-manufacturers-continued-losses-prompt-industry-action/; https://ca.finance.yahoo.com/news/chinas-solar-giants-quietly-shed-045152284.html; https://asia.nikkei.com/business/energy/china-s-solar-giants-warn-of-record-5bn-loss-despite-anti-price-war-bid; https://www.reuters.com/sustainability/climate-energy/china-solar-industrys-losses-narrowing-overcapacity-push-association-says-2025-12-18/; https://www.bloomberg.com/news/articles/2025-08-26/china-s-solar-industry-signals-hope-despite-more-heavy-losses; https://www.pv-tech.org/chinas-top-four-solar-manufacturers-suffer-1-54-billion-losses-h1-2025/

(Images: here; here; data here)